HOW TO CHANGE OIL ON 2017 NISSAN MAXIMA

- Place chocks at rear wheels and raise the front end using a hydraulic jack to its highest point. At the front garage jack point on the Maxima, which is immediately behind the plastic undercover under the bumper , make sure you use the right side dimple with the hole, NOT THE LEFT DIMPLE (the right side sticks out a bit more and is in the center of the sub frame). See where the “3” points to in the diagram below.

- Push Rhino ramps under wheels aligning them precisely and as far back as they can go.

- Lower the hydraulic jack so wheels rest on the Rhino ramps. The wheels will be at an angle and the car will have to be driven further up the ramps.

- Drive the car further up the ramps to horizontal plane almost to the stops. Be cautious when doing this as Maxima has lots of power and you don’t want to shoot past the end of the ramps. Use throttle and brake using two feet alternately to get car up to the end of the ramps. An assistant (off to the side) can help guide you to proper spot.

- Push wheels chocks further in to block wheels as the car moved forward since Step 1).

- Remove three plastic clips under the oil pan, using two flat head screwdrivers to pry up the center pin, with each screwdriver on opposite sides. This prevents damaging the soft plastic. Once the center part has been pried out, you can pull the center completely out with your fingers. Hold the center pin of the plastic clip and pull out the whole clip. You may need to wiggle a bit or hold the center part of the clip and push it in and out a few times and it will just come out as one entire piece. This process causes NO damage to the plastic clips. Keep in mind that the pin closest to the front of the car is smaller than the other two (small replacement clips are Nissan part #01553-09321 or 01553-2DR9A and large clips are Nissan part #01553-09241).

- From underneath the car you can then remove the oil filter plastic cover (“right engine protector” in owner’s manual) inside the front passenger side wheel well. There are 5 similar plastic center push pins to remove. Keep in mind that the bottom pin under the sub-frame is larger (Nissan part ##01553-09241) than the other 4 pins (Nissan part #01553-05323) inside the wheel well. Also, when replacing this plastic wheel well cover, make sure the bottom right part of the cover (when looking at it) fits inside the other plastic wheel well plastic so minimal debris gets under the plastic.

- Remove the oil fill cap and dipstick.

- Using a 14 mm socket, remove the oil pan drain plug. Let drain for a while until single drops are dropping.

- Remove the oil filter using a strap wrench, or a fluted specialty oil wrench (65-67 mm and 14 flutes) and let the oil flow to single drops. You will have to clean up the spill on top of the sub-frame as there is no way to remove the filter without spillage.

- Make sure the oil pan plug is clean and that you use a new brass washer. You may get away with using the old washer, but they deform quite a bit with installation and reusing it may cause oil leakage (Nissan part #11026-JA00A- $1.25 at my Nissan dealer). Tighten to 25 ft lb. DO NOT TIGHTEN FURTHER as you may crack the oil pan.

- Apply some fresh oil with your finger around the new oil filter rubber/silicone gasket.

- Clean off the top and bottom of the sub-frame and the oil filter sealing surface on the oil pan and screw on the new oil filter by hand until slight resistance is felt. Make a mark on the filter and turn it an additional 2/3 to ¾ turn or 10.8-15.1 ft lb. Do not tighten further as it may be hard to remove the oil filter later.

- RE-INSTALL THE OIL DIPSTICK before you fill 4.8 litres or 5 1/8 quart of 0W-20 oil (this assumes a complete oil and filter change and relatively thorough draining of the oil pan). If you do not install the dipstick first, oil may come out the dipstick hole.

- Run the engine for a little bit to make sure the oil gets distributed and shut the engine off to check the oil drain plug and oil filter for any leaks.

- If no leaks, re-install the oil pan plastic cover first with the proper clips (3).

- Re-install the oil filter plastic cover in the wheel well with the proper clips (5).

- Lower the vehicle and run the engine for a bit and re-check the dipstick to ensure the proper oil level. Add more oil if necessary, but DON’T OVERFILL.